PGV 为位置导引视觉系统(Position Guided Vision)

请求

- 编号: 1017 (0x03F9)

- 名称: robot_status_pgv_req

- 描述: 查询二维码数据(PGV)数据

- JSON 数据区: 无

请求示例

5A 01 00 01 00 00 00 00 03 F9 00 00 00 00 00 00

响应

- 编号: 11017 (0x2B09)

- 名称: robot_status_pgv_res

- 描述: 查询二维码数据(PGV)数据的响应

- JSON 数据区: 见下表

| 字段名 | 类型 | 描述 | 可缺省 |
| --- | --- | --- | --- |
| pgvs | array[object] | 二维码识别数据, 数据示例见下文 | 是 |
| ret_code | number | API 错误码 | 是 |
| create_on | string | API 上传时间戳 | 是 |
| err_msg | string | 错误信息 | 是 |

object 形式如下:

主要关注 device_address（PGV 设备的 id）和 tag_value（二维码标签的数据）

{

// PGV 设备的 id

"device_address": 1,

"error_code": 0,

"header": {

"data_nsec": "1750000088331",

"frame_id": "",

"pub_nsec": "0",

"seq": "0"

},

"is_DMT_detected": true,

"is_absolute_X_pos_valid": true,

"pgv_info": {

"func": 1,

"r": 0,

"x": 0,

"y": 0,

"z": 0

},

"tag_diff_angle": 1744,

"tag_diff_x": 256,

"tag_diff_y": 47,

// 二维码标签的数据

"tag_value": 1,

"warning_code": 0

}

响应示例

如下为两个 PGV 设备的数据:

{

"pgvs": [

{

"device_address": 0,

"error_code": 0,

"header": {

"data_nsec": "1749997924588",

"frame_id": "",

"pub_nsec": "0",

"seq": "0"

},

"is_DMT_detected": false,

"is_absolute_X_pos_valid": false,

"pgv_info": {

"func": 1,

"r": 0,

"x": 0,

"y": 0,

"z": 0

},

"tag_diff_angle": 0,

"tag_diff_x": 0,

"tag_diff_y": 0,

"tag_value": 0,

"warning_code": 0

},

{

"device_address": 1,

"error_code": 0,

"header": {

"data_nsec": "1749997924588",

"frame_id": "",

"pub_nsec": "0",

"seq": "0"

},

"is_DMT_detected": true,

"is_absolute_X_pos_valid": true,

"pgv_info": {

"func": 2,

"r": 0,

"x": 0,

"y": 0,

"z": 0

},

"tag_diff_angle": 1790,

"tag_diff_x": 54,

"tag_diff_y": 3,

"tag_value": 1,

"warning_code": 0

}

]

}

为方便阅读，这里 JSON 数据采用了展开的形式, 实际使用中为减少数据流 JSON 数据区是紧凑的, 不会有多余的空格与换行, 后文中同理。