查询机器人支持标定列表

## 请求

编号: 1509

名称: ROBOT_STATUS_CALIB_SUPPORT_LIST_REQ

描述: 查询机器人支持标定列表

## 响应

编号: 11509

名称: ROBOT_STATUS_CALIB_SUPPORT_LIST_RES

描述: 查询机器人支持标定列表的响应

JSON 数据区: 见下

JSON { "calib_list":[ { "calib_type":1, //标定类型 "device_name":"laser1", //设备名 "is_auto":true, //false:需要人工干预 true:不需要人工干预 "text":"laser1", //提示字段 "desc":"calib laser1", //提示字段 "is_calibrated":false, //false:未标定 true:已标定 "available":true, //是否可标定，true:是 false:否 "priority_type":0, //标定优先级，值越小优先级越高 "layout":"SinSteer_1", //标定项后面显示的提示消息 "hasPlot":true //是否有标定图，true:有 false:无 } ] }

#### 注1 标定类型

JSON //Movement types enum CalibMoveType { NoCalibMove = 0,//No calib task LaserOdomCalib = 1,//Laser(Loc sensor)-Odom calibration IMUInitCalib = 2,//IMU initial calibration LocCameraCalib = 3,//PGV external parameters calibration RealSenseManualCalib = 4,//Manual calib type for RealSense-435-USB BiSteerACOffsetCalib = 5,//(AC) Steering angle offset calibration for bisteer vehicles LaserCalibBasedonOdom = 6,//Calibration of external parameters of laser based on accurate odometry PGVCalib = 7,//PGV calibration based on odometer TempSteerOffsetCalib = 8,//temporal test for measuring steer offset SteerRangeCalib = 9,//Steer angle range calibration OmniRollerAngleCalib = 10,//Roller angle (the angle from wheel axis to roller axis) calibration SteerPiecewiseCalib = 11,//Piecewise laser-odometer calibration for steer vehicle RepeatedLaserOdomCalib = 12,//repeatedly execute laser-odom calibration to collect data CalibInteractTest = 13,//test for interact mechanism in Message_CalibStatus CameraManualCalib =14,//manual calibration for camera (directly call service of MultiDcamera) BiSteerDCOffsetCalib=15,//(DC) Steering angle offset calibration for bisteer vehicles PGVOdomCalib=16,//PGV-odometer calibration for differential vehicles with only PGV as localizaiton sensor PGVRangeCalib=17,//PGV x/yRange calibration ForkTestMove=18,//Test move for fork characteristics identification under certain load GNSSCalib=19,//External parameters calibration for GNSS device ArmCameraHandEyeCalib=20,//Hand-eye calibration for cameras mounted on manipulator CameraInternalCalib=21,//Internal parameters calibration for cameras mounted on manipulator IMUCalibNoRotInPlace=22,//IMU initial calibration for vehicles that CANNOT rotate in place OmniLaserOdomCalib=23,//Laser-Odom calibration for omni vehicles OdomLocCompareTest=24,//Odometer-localization trajectory comparision test ChargerManualCalib=25,//Manually input the position correction of the charger OpticalOdomCalib=26,//Optical-odometer calibration BiSteerLaserOdomCalib = 27, //BiSteer Laser-odom calibration ForceSensorCalib=28, //Force sensor offset calibration IRCameraExtrinsicCalib = 29, // IR Camera Extrinsic calibration SinSteerYAxisManualCalib = 30,//Single steer Y direction coordinate manual calibration SinSteerOffsetFileOverwriteManualCalib = 31,//Single steer offset file overwrite manual calibration SinSteerZeroOffsetAutoCalib = 32,//Single steer offset when at 0 degree automatic calibration SinSteerXAxisManualCalib = 33,//Single steer X direction coordinate manual calibration SinSteerOffsetFileManualCalib=34,//Single steer set or get steer offset file manual calibration SinSteerLaserYawManualCalib = 35,//Single steer laser Yaw manual calibration SinSteerReachdistManualCalib = 36,//Single steer Reachdist manual calibration SinSteerLaserXYManualCalib = 37,//Single steer laser XY manual calibration SinSteerCheckManualCalibResults = 38,//Single steer check manual calibration results ObsLaserCalib=39,//Obstacle avoidance laser calibration SinSteerWheelbaseShiftCalib=40,//Single steer wheelbase shift calibration };

#### 注2 标定优先级

JSON //Calibration priority enum CalibPriotity { BasicHardwareCalib = 0,//SteerRangeCalib OdometerCalib = 2,//LaserOdomCalib, BiSteerOffsetCalib, OmniRollerAngleCalib, SinSteerManualCalib SubOdometerCalib = 3,//for BiSteerDCOffsetCalib AuxOdometerCalib = 4,//TempSteerOffsetCalib AuxSensorCalib = 5,//ArmCameraInternalCalib ExternParaCalib = 6,//IMUInitCalib, LocCameraCalib, RealSenseManualCalib, LaserCalibBasedonOdom, PGVCalib, ArmCameraHandEyeCalib TestCalib = 100//TempRotCenterCal, RepeatedLaserOdomCalib };