上传手柄自定义绑定事件

## 请求

编号: 4470 (0x1176)

名称: robot_config_joystick_bind_keymap_req

描述: 上传手柄自定义绑定事件

JSON 数据区: 见下表

| 字段名 | 类型 | 描述 | 可缺省 |
| --- | --- | --- | --- |
| keymap | json | 显示可绑定事件和绑定结果 | 否 |

### 请求示例

JSON { "bindListPress": { // 当按键按下 "B": { // 示例只展示一个按键 "combination": { // 和 LB 使能键配合使用时 "action": "noBind", // 绑定动作 "actions": { // 列表，表示可绑定动作 "cancelNavigation": { "desc": "Cancel navigation" }, "confirmLoc": { "desc": "Confirm location" }, "gotoInitPoint": { "desc": "Go to init point" }, "gotoNextPoint": { "desc": "Go to next point" }, "gotoPreviousPoint": { "desc": "Go to previous point" }, "noBind": { "desc": "No bind" }, "pauseOrContinueNavigation": { "desc": "Pause or continue navigation" }, "setDI": { "desc": "Set DI" } } }, "single": { // 按键单独使用 "action": "gotoNextPoint", "actions": { "cancelNavigation": { "desc": "Cancel navigation" }, "confirmLoc": { "desc": "Confirm location" }, "gotoInitPoint": { "desc": "Go to init point" }, "gotoNextPoint": { "desc": "Go to next point" }, "gotoPreviousPoint": { "desc": "Go to previous point" }, "noBind": { "desc": "No bind" }, "pauseOrContinueNavigation": { "desc": "Pause or continue navigation" }, "setDI": { "desc": "Set DI" } } } } }, "bindListRelease": { // 当按键抬起 "B": { "combination": { "action": "noBind", "actions": { "cancelNavigation": { "desc": "Cancel navigation" }, "confirmLoc": { "desc": "Confirm location" }, "gotoInitPoint": { "desc": "Go to init point" }, "gotoNextPoint": { "desc": "Go to next point" }, "gotoPreviousPoint": { "desc": "Go to previous point" }, "noBind": { "desc": "No bind" }, "pauseOrContinueNavigation": { "desc": "Pause or continue navigation" }, "setDI": { "desc": "Set DI" } } }, "single": { "action": "noBind", "actions": { "cancelNavigation": { "desc": "Cancel navigation" }, "confirmLoc": { "desc": "Confirm location" }, "gotoInitPoint": { "desc": "Go to init point" }, "gotoNextPoint": { "desc": "Go to next point" }, "gotoPreviousPoint": { "desc": "Go to previous point" }, "noBind": { "desc": "No bind" }, "pauseOrContinueNavigation": { "desc": "Pause or continue navigation" }, "setDI": { "desc": "Set DI" } } } } }, "version": 1.01 }

## 响应

编号: 14021 (0x3886)

名称: robot_config_joystick_bind_keymap_res

描述: 上传手柄自定义绑定事件的响应

JSON 数据区: 见下

| 字段名 | 类型 | 描述 | 可缺省 |
| --- | --- | --- | --- |
| ret_code | number | API 错误码 | 是 |
| create_on | string | API 上传时间戳 | 是 |
| err_msg | string | 错误信息 | 是 |

### 响应示例

JSON { "create_on": "2024-05-07T13:54:38.796+0800", "ret_code": 0 }