查询机械臂状态

## 请求

编号: 1669 (0x0685)

名称: robot_status_armstatus_req

描述: 查询机械臂状态

JSON 数据区: 无

### 请求示例

无

## 响应

编号: 11669 (0x2D95)

名称: robot_status_armstatus_res

描述: 查询机械臂状态的响应

JSON 数据区: 见下

响应数据是通过Message_ArmInfo序列化为json得来，具体字段以message_arminfo.proto中为准。

序列化为json时，使用的序列化参数如下

C++ std::string j_str; google::protobuf::util::JsonOptions opt; opt.always_print_enums_as_ints = true; opt.always_print_primitive_fields = true; opt.preserve_proto_field_names = true; google::protobuf::util::MessageToJsonString(msg, &j_str, opt);

### 响应示例1：

JSON { "enabled": false, "safety_mode": 0, "arm_mode": 0, "DI": [], "DO": [], "robot_status_bits": [], "safety_status_bits": [], "taskId": 0, "task_status": 0 }

### 响应示例2：

JSON { "DI": [ ], "DO": [ ], "arm_mode": 7, "base": { "angle": -1.6007002035724085, "current": 0, "temperature": 0, "velocity": 0, "voltage": 0 }, "create_on": "2023-04-25T17:20:34.202+0800", "elbow": { "angle": -2.2029998938189905, "current": 0, "temperature": 0, "velocity": 0, "voltage": 0 }, "enabled": true, "endpos": { "rx": -0.00039174698543340767, "ry": 0.9998434776685855, "rz": 0.012478298683965675, "w": 0.012536297692776333, "x": -0.1799259141475472, "y": -0.6068697551627643, "z": 0.23078910206725664 }, "load": { "device_name": "", "fx": 0, "fy": 0, "fz": 0, "mx": 0, "my": 0, "mz": 0 }, "ret_code": 0, "robot_status_bits": [ true, false, false, false ], "safety_mode": 1, "safety_status_bits": [ true, false, false, false, false, false, false, false, false, false, false ], "scan": { "data": "", "name": "", "scan_status": 1 }, "shoulder": { "angle": -1.7271001974688929, "current": 0, "temperature": 0, "velocity": 0, "voltage": 0 }, "taskId": 0, "task_status": 2, "wrist1": { "angle": -0.8079999128924769, "current": 0, "temperature": 0, "velocity": 0, "voltage": 0 }, "wrist2": { "angle": 1.5951000452041626, "current": 0, "temperature": 0, "velocity": 0, "voltage": 0 }, "wrist3": { "angle": -0.03099996248354131, "current": 0, "temperature": 0, "velocity": 0, "voltage": 0 } }

### 说明：

示例2针对针对UR机械臂上报的状态信息。其中机械臂状态的字段含义如下：

C++ // Bits 0-3: // ¦ Is power on ¦ Is program running // ¦ Is teach button pressed ¦ Is power button pressed std::vector<bool> armStatus; // Bits 0-10: // ¦ Is normal mode ¦ Is reduced mode // ¦ Is protective stopped ¦ Is recovery mode // ¦ Is safeguard stopped ¦ Is system emergency stopped // ¦ Is robot emergency stopped ¦ Is emergency stopped // ¦ Is violation ¦ Is fault ¦ Is stopped due to safety std::vector<bool> armSafeStatus; // * -1 = ROBOT_MODE_NO_CONTROLLER // * 0 = ROBOT_MODE_DISCONNECTED // * 1 = ROBOT_MODE_CONFIRM_SAFETY // * 2 = ROBOT_MODE_BOOTING // * 3 = ROBOT_MODE_POWER_OFF // * 4 = ROBOT_MODE_POWER_ON // * 5 = ROBOT_MODE_IDLE // * 6 = ROBOT_MODE_BACKDRIVE // * 7 = ROBOT_MODE_RUNNING // * 8 = ROBOT_MODE_UPDATING_FIRMWARE uint32_t armMode; // 1 : normal mode // 2 : reduced mode // 3 : recovery mode (protective stoped) uint32_t armSafeMode;

字段为[ ]的表示数据为空，该字段数据暂不上报。

除UR机械臂外的其他机械臂的状态字段数据暂未上报。