3D 地图格式

## 一、文件格式

3d地图后缀为smap，实际是一个zip压缩包，包含三个文件：

info.json

0.smap

0.3dsmap

用于描述地图及其元信息。三个文件均应位于压缩包根目录。

## 二、文件说明

### 1. info.json

类型：JSON 格式文本文件

作用：存放数据包基本信息

json数据：

| 字段名 | 类型 | 描述 | 可缺省 |
| --- | --- | --- | --- |
| zipName | string | 20250606165847531-3D | 否 |

示例：

JSON { "zipName": "20250606165847531-3D" }

### 2. 0.smap

类型：JSON 格式文本文件

内容：2D格式地图，参见附录C: 地图格式

### 3. 0.3dsmap 文件

类型：ProtoBuf 序列化的二进制文件

内容：3D地图信息

格式：Message_Map3D

Bash syntax = "proto3"; package rbk.protocol; import "google/protobuf/wrappers.proto"; import "message_header.proto"; import "message_imu.proto"; import "message_gnss.proto"; import "message_localization.proto"; message Message_MapLogData { double robot_odo_x = 1; double robot_odo_y = 2; double robot_odo_w = 3; repeated double laser_beam_dist = 4; repeated double laser_beam_angle = 5; repeated double rssi = 6; Message_Header header = 7; } message Message_MapOdo { double timestamp = 1; float odo_x = 2; float odo_y = 3; float odo_w = 4; float odo_vx = 5; float odo_vy = 6; float odo_vw = 7; } message Message_MapLogData3D { double timestamp = 1; repeated float x = 2; repeated float y = 3; repeated float z = 4; repeated uint32 intensity = 5; repeated uint32 timeoffset = 6; repeated uint32 ring = 7; repeated bytes data = 8; repeated float firstAzimuth = 9; repeated float secondAzimuth = 10; } message Message_MapLog { double laser_pos_x = 1; double laser_pos_y = 2; double laser_pos_z = 3;//由于版本原因里面设置是激光安装yaw角，取激光高度数据从laser_install_height double laser_step = 4; double laser_range_max = 5; repeated Message_MapLogData log_data = 6; string laser_name = 7; double laser_install_height = 8; repeated Message_MapOdo odometer = 9; repeated Message_MapLogData3D log_data3d = 10; double laser_install_yaw = 11; double laser_install_pitch = 12; double laser_install_roll = 13; repeated Message_IMU imu_data = 14; repeated Message_GNSS gnss_data = 15; uint32 lasertype = 16; // lasertype = 1---robosense 16 lasertype = 2----robosense helios lasertype = 3----velodyne 16 float factor = 17; repeated float azimuthcorrection = 18; repeated float verticalcorrection = 19; repeated Message_AllGNSS all_gnss_data = 20; // 支持多个天线设备 repeated Message_Localization localization_data = 21; } message Message_MapProperty { string key = 1; string type = 2; //如果type = "json"时,表示 string_value中是个json的string 方便以后扩展 bytes value = 3; // for backward compatibility oneof oneof_value { string string_value = 4; bool bool_value = 5; int32 int32_value = 6; uint32 uint32_value = 7; int64 int64_value = 8; uint64 uint64_value = 9; float float_value = 10; double double_value = 11; bytes bytes_value = 12; } string tag = 13; //调度场景专用表示该key作用于指定机器人组(例如: "group:test_1,group1:test_2,group2")如果为空表示作用于所有机器人 } message Message_MapPos { double x = 1; double y = 2; double z = 3; } message Message_MapRSSIPos { double x = 1; double y = 2; } message Message_ReflectorPos { string type = 1; double width = 2; double x = 3; double y = 4; uint32 creation_method = 5;//创建方式:creation_method(0:手动添加 1:建图时生成 2:修改过) } message Message_LiveRefPos { repeated Message_ReflectorPos ref_pos = 1; } message Message_tagPos { uint32 tag_value = 1; double x = 2 ; double y = 3; double angle = 4 ; bool is_DMT_detected = 5; double z = 6; double qx = 7; double qy = 8; double qz = 9; double qw = 10; double variance = 11; string class_name = 12; //2DTAG,3DTAG repeated Message_MapProperty property = 13; } message Message_MapLine { Message_MapPos start_pos = 1; Message_MapPos end_pos = 2; } message Message_MapHeader { string map_type = 1; //type of map: 2D-Map or 3D-Map string map_name = 2; //name of map: the map file name Message_MapPos min_pos = 3; Message_MapPos max_pos = 4; double resolution = 5; //unit: m string version = 8; } message Message_MapAttribute { string description = 1; // description of the this class uint32 color_pen = 2; uint32 color_brush = 3; uint32 color_font = 4; } message Message_AdvancedPoint { string class_name = 1; //the class_name from the Message_AdvancedObjectDefine string instance_name = 2; // the name of this instance Message_MapPos pos = 3; double dir = 4; repeated Message_MapProperty property = 5; // write in json bool ignore_dir = 6; bytes desc = 8; Message_MapAttribute attribute = 10; } message Message_AdvancedLine { string class_name = 1; //the class_name from the Message_AdvancedObjectDefine string instance_name = 2;// the name of this instance Message_MapLine line = 3; repeated Message_MapProperty property = 4; // write in json bytes desc = 8; Message_MapAttribute attribute = 10; } message Message_AdvancedCurve { string class_name = 1; //the class_name from the Message_AdvancedObjectDefine string instance_name = 2;// the name of this instance Message_AdvancedPoint start_pos = 3; Message_AdvancedPoint end_pos = 4; Message_MapPos control_pos1 = 5; Message_MapPos control_pos2 = 6; repeated Message_MapProperty property = 7; // write in json bytes desc = 8; Message_MapPos control_pos3 = 9; Message_MapPos control_pos4 = 10; repeated Message_Device devices = 12; Message_MapAttribute attribute = 15; } message Message_AdvancedArea { string class_name = 1;//the class_name from the Message_AdvancedObjectDefine string instance_name = 2;// the name of this instance repeated Message_MapPos pos_group = 3; // usually four lines in a group double dir = 4; repeated Message_MapProperty property = 5; // write in json bytes desc = 8; repeated Message_Device devices = 10; Message_MapAttribute attribute = 15; } message Message_VirtualLineList { repeated Message_MapLine virtual_map_line = 1; } message Message_LaserDevice { uint32 id = 1; repeated Message_MapPos laser_margin_pos = 2; } message Message_Device { string model_name = 1; repeated Message_LaserDevice laser_devices = 5; repeated double ultrasonic_dist = 6; // dist = -1 means not used repeated double fallingdown_dist = 7; // dist = -1 means not used } message Message_PatrolRouteStation { string id = 1; } message Message_PatrolRoute { string name = 1; repeated Message_PatrolRouteStation station_list = 2; google.protobuf.DoubleValue max_speed = 4; google.protobuf.DoubleValue max_acc = 5; google.protobuf.DoubleValue max_rot = 6; google.protobuf.DoubleValue max_rot_acc = 7; bytes desc = 8; google.protobuf.DoubleValue max_dec = 9; google.protobuf.DoubleValue max_rot_dec = 10; } message Message_Primitive { string class_name = 1; //the class_name from the Message_AdvancedObjectDefine string instance_name = 2;// the name of this instance Message_AdvancedPoint start_pos = 3; Message_AdvancedPoint end_pos = 4; repeated Message_MapPos control_pos_list = 5; repeated Message_MapProperty property = 6; // write in json bytes desc = 7; Message_MapAttribute attribute = 8; } message Message_ExternalDevice { string class_name = 1; //the class_name from the Message_AdvancedObjectDefine string instance_name = 2;// the name of this instance bool is_enabled = 3; repeated Message_MapProperty property = 4; // write in json bytes desc = 5; Message_MapAttribute attribute = 6; } message Message_BinLocation {//库位 string class_name = 1; //库位class_name string instance_name = 2; //库位唯一值 string group_name = 3; //库区名称 //弃用，已移到 message_rds_scene.proto中 string point_name = 4; //关联的站点名称 Message_MapPos pos = 5; repeated Message_MapProperty property = 6; //拓展属性 bytes desc = 7; Message_MapAttribute attribute = 8; } message Message_BinLocations {//库位数组 (x,y相同的库位放到一个数组中) repeated Message_BinLocation bin_location_list = 1;//库位 } message Message_Map { string map_directory = 1; Message_MapHeader header = 2; repeated Message_MapPos normal_pos_list = 3; //普通点 repeated Message_MapLine normal_line_list = 4; //普通线 repeated Message_MapPos normal_pos3d_list = 5; //普通3d点云 repeated Message_AdvancedPoint advanced_point_list = 6; //站点 repeated Message_AdvancedLine advanced_line_list = 7; //禁行线 repeated Message_AdvancedCurve advanced_curve_list = 8; //连接线 repeated Message_AdvancedArea advanced_area_list = 9; //高级区域 repeated Message_PatrolRoute patrol_route_list = 10; // repeated Message_MapRSSIPos rssi_pos_list = 11; //反光板点 repeated Message_ReflectorPos reflector_pos_list = 12; //反光板/反光柱 repeated Message_tagPos tag_pos_list = 13; //二维码 repeated Message_Primitive primitive_list = 14; //图元 repeated Message_ExternalDevice external_device_list = 15; //外部设备 repeated Message_BinLocations bin_locations_list = 16; //库位 repeated Message_MapProperty user_data = 100; //支持地图中自定义一些属性 } message Message_Map3D { string map_directory = 1; Message_MapHeader header = 2; repeated Message_MapPos normal_pos3d_list = 3; // 普通3d点云 FeatureMap3D feature_map_3d = 4; // 3D 特征地图 } // =================== 多线激光特征定位相关 begin message Vec3f { float x = 1; float y = 2; float z = 3; } message Vec3i { int32 x = 1; int32 y = 2; int32 z = 3; } message FeatureMapParams { // 激光测量距离的标准差，单位m float ranging_sigma = 1; // 激光测量角度的标准差，单位deg float angle_sigma = 2; // 体素最大的边长，单位m float max_voxel_size = 3; // 体素的最大层数 uint32 max_layer = 4; // 不再更新平面协方差的点数门限 uint32 cov_fixed_pts_num = 5; // 不再更新平面的点数门限 uint32 plane_fixed_pts_num = 6; // 有效平面协方差最小特征值的门限 float plane_min_eigen_value = 7; // 体素每层对应评估平面最少所需的点数 repeated int32 each_layer_least_pts_num = 8; } message FeatureMapPlane { // 平面的重心 Vec3f center = 1; // 平面法向量 Vec3f normal = 2; //平面方程 Ax + By + Cz + D = 0 float d = 3; // 评估的平面半径 float radius = 4; // 平面协方差 6x6 repeated float plane_cov = 5; } message OctoTree { // 对应的平面ID uint32 plane_id = 1; // 子节点的序列，依次从最外层到当前层 // -1 表示当前层下无其他子节点 repeated int32 child_id_list = 2; } // 当前位置的八叉树容器， 可能包含很多很多子节点 message OctoTrees { repeated OctoTree octo_tree = 1; } message FeatureMap3D { FeatureMapParams params = 1; repeated FeatureMapPlane planes = 2; repeated OctoTrees octo_trees = 3; // 最外层的八叉树位置 repeated Vec3i voxel_locs = 4; } // end 多线激光特征定位相关 ===================